ALASKA FISHERIES SCIENCE CENTER	STANDARD OPERATING PROCEDURES

FOR HAZARDOUS CHEMICALS

July 29, 2026

3

1

# C

3

AFA Fixative: Field Use (50% Distilled water, 30% Ethyl Alcohol, 10% Formalin, 10% Acetic Acid)

| #1 PROCESS | Use of AFA in the field or on research vessels for fixation purposes. |
| --- | --- |
| #2 HAZARDOUS CHEMICAL NOTES | 30% Ethyl Alcohol (EtOH): Flammable/Nervous System Depressant 10% Formaldehyde: Carcinogen, harmful b absorption through skin/eyes, inhalation and ingestion. Glacial Acetic Acid: Poison! Corrosive. Liquid and mist cause severe burns to all body tissue. May be fatal if swallowed. Harmful if inhaled. |
| #3 PERSONAL PROTECTIVE EQUIPMENT | Nitrile or rubber gloves, goggles/safety glasses, apron or rain gear, rubber boots. Get a co-worker to stand by as an observer. |
| #4 WORK AREA PRECAUTIONS | All work should be performed outside in plenty of fresh air or in lab with good ventilation and accessible water hoses. Advise the Field Party Chief or a co-worker of your intent to work with the solution. |
| #5 SPECIAL HANDLING PROCEDURES & STORAGE REQUIREMENTS | 30% EtOH: Store in cool, dry place away from strong oxidizing agents, halides, alkali metals, silver salts and metal hydrides. 10% Formaldehyde: Store in cool, dry place away from strong oxidizing or reducing agents Glacial Acetic Acid: Store in cool, dry place away from metals, water, sulfuric acid, strong bases, amines, carbonates, alkalies, formaldehyde, metal oxides, acetic anhydride, vinyl acetate, mercuric sulfate, and calcium phosphide. |
| #6 SPILL & ACCIDENT PROCEDURES/FIRST AID | REMOVE ALL SOURCES OF POSSIBLE IGNITION! Alert bystanders of hazard. 30% EtOH: If spill occurred on outside deck of ship, and quantity of EtOH spilled is less than 5 gallons, you may dilute spill and wash chemical from deck using a water hose. Use caution and avoid splashing and spreading chemical. Ventilate spill area. 10% Formaldehyde: For small spills spray on Formalex and wipe up with Formalex pads. For larger spills, circle the spill with Spill-X for formaldehyde to contain spill to one area. Fill the circle with Spill-X or Formalex and sweep or wipe up. Wear protective clothing and respirator. Glacial Acetic Acid: For small spills sprinkle with Spill-X for acids/corrosives and sweep up. For larger spills, circle the spill with Spill-X for acids/corrosives to contain spill to one area. Fill the circle with Spill-X and sweep or wipe up. ***For LARGE spills or extreme exposure of Glycerol Alcohol, contact FPC & Skipper*** In case of skin/eye contact: flush with running water for at least 15 min. In case of ingestion: give copious amounts of water and induce vomiting. In case of inhalation: move to fresh air. If difficulty breathing, give oxygen. If not breathing, administer CPR and call a physician. |
| #7 WASTE DISPOSAL | All recovered material should be packaged, labeled, transported and disposed of, or reclaimed in conformance with applicable laws & regulations |
| #8 SPECIAL PRECAUTIONS FOR ANIMAL USE | AFA is used as a preservative for animal and plant tissue. Make certain that whole animals are dead prior to being placed in fixative. |
| #9 APPROVAL REQUIRED | Users must receive specific physical and health hazard information and safe laboratory work practices training from their supervisor. MSDSs and NIOSH Pocket Guide are available. |
| #10 DECONTAMINATION | See #6 and #7 above. |
| #11 DESIGNATED AREA | Special storage is required – must be stored on deck, not in a confined space and away from living quarters. Make sure that container is well labeled. |