ALASKA FISHERIES SCIENCE CENTER	STANDARD OPERATING PROCEDURES

FOR HAZARDOUS CHEMICALS

July 30, 2026

3

0

# C

0

10% Formalin Solution: Field Use

| #1 PROCESS | Use of 10% formalin in the field or on research vessels. |
| --- | --- |
| #2 HAZARDOUS CHEMICAL NOTES | 10% formalin. Carcinogen; harmful by absorption through skin/eyes, inhalation and ingestion |
| #3 PERSONAL PROTECTIVE EQUIPMENT | Nitrile or rubber gloves, goggles/safety glasses, apron or rain gear, rubber boots. Get a co-worker to stand by as an observer. |
| #4 WORK AREA PRECAUTIONS | All work should be performed outside in plenty of fresh air or in a laboratory with good ventilation and accessible water hoses. While working, have Formalex nearby or know where it is. Advise the Field Party Chief or a co-worker of your intent to work with the solution. Fume hoods when present, must be tested and certified regularly. |
| #5 SPECIAL HANDLING PROCEDURES & STORAGE REQUIREMENTS | Store in cool dry place away from strong oxidizing or reducing agents. |
| #6 SPILL & ACCIDENT PROCEDURES/FIRST AID | For small spills spray on Formalex and wipe up with Formalex pads. For larger spills, circle the spill with Spill-X for formaldehyde to contain spill to one area. Fill the circle with Spill-X or Formalex and sweep, wipe up, or hose down. For spills on deck, attempt to pour Formalex on spill before hosing down deck. This reduces vapors and creates a safer environment for coworkers (ie. neutralizes splash). In case of skin/eye contact: flush with running water for at least 15 min. Toxic, exposure may cause irritation and possible burns. In case of ingestion: give large amounts of water and induce vomiting. Ingestion may cause severe abdominal pain, vomiting, headache, diarrhea. In case of inhalation: move to fresh air. Exposure may cause sore throat, coughing and shortness of breath. If difficult breathing give oxygen. If not breathing, administer CPR. Call a physician. ***For large spills or extreme exposure of formalin contact FPC & Skipper*** |
| #7 WASTE DISPOSAL | Pour waste formalin into a container and mix with Neutralex. Stir and let sit for 15 minutes and follow directions on package and wash down the scupper with hose. |
| #8 SPECIAL PRECAUTIONS FOR ANIMAL USE | 10% formalin is used as a preservative for animal and plant tissue. Make certain that whole animals are dead or anesthetized prior to being placed in fixative. |
| #9 APPROVAL REQUIRED | Users must receive specific physical and health hazard information and safe laboratory work practices training from their supervisor. MSDSs and NIOSH Pocket Guide are available. |
| #10 DECONTAMINATION | See #6 and #7 above. |
| #11 DESIGNATED AREA | Special storage is required – must be stored on deck, not in a confined space and away from living quarters. Make sure that container is well labeled. |