In-situ protocol : Antonio, 2001

Day 1

- Adjust oven to 37 oC : during the permeabilization & prehybridization steps, then 53oC during hybridization (O/N)

- Adjust oven to 42 C for washing step in the next day and put reagents in

the oven O/N

## PCR for RLP from Abalone

# Reagents		50 l reaction (recommend 25 ul Rxn)

sdw			24.6 l

MgCl			1.5

Buffer 			5

TMAC			5

DNTPs			4

BSA 10 mg/ml		0.5

Primers

RA5-1		2

RA3-6		2

Taq			0.2

DNA template		3

# Cycling conditions

1 = 95°C for 5:00

2 = 94°C for 1:00

3 = 50°C for 0:30 (62°C)

4 = 72°C for 0:30

5 = Go to 2, 39 times

6 = 72°C for 7:00

7 = 16°C forever

## Labeling by PCR using PCR DIG Probe Synthesis Kit

Roche Applied Science: Cat No. 11 636 090 910

Add the following components to a sterile microfuge tube on ice:

Reagent				Volume (l)

sdw					29.25

PCR buffer (vial 3)			5

PCR DIG mix (vial 2)			5

Primers (upstream, downstream)

RA 5-1  (20 pml/l)		2.5

RA 3-6	 (20 pml/l)		2.5

Enzyme mix (vial 1)			0.75

Template DNA

(1:100 dilution of RLP plasmid)	5

Note: Conditions for labeling are the same as conventional PCR outline above. Always include one reaction without DIG labeling (i.e. regular PCR reaction containing RLP DNA template).

Important: Before using the DIG labeled probe, denature the probe at 95C for 3 minutes and immediately place on ice for about 30 min to separate the double stranded DNA. Store at –20 or –70C until use.

Tissue Deparaffinization ~ 1.20 hr

1.  Deparaffinize with 3 changes of Xylene for 10 min each.

2.  Hydrate with graded ethanol series of (100,100, 95, 80, 70, and 50%), for 3 min

each.

3.  Then rinse slides in sterile dH20.

4.  Equilibrate tissue sections in Tris buffer pH 7.2

0.2 M Tris-HCl

2.0 mM CaCl

Permeabilization of tissues ~ 1.20 hr

1.  Add tissues with 50 ug/ml Proteinase K in Tris-buffer for 45 min at 37 oC.

2.  Rinse tissues with PBS: 3x10 min.

Prehybridization ~ 2 hr

1.  Prepare prehybridization buffer (Total volume = 1 mL):

0.51 mL deionized formamide

0.20 mL 20x SSC

0.05 mL heat-denatured sperm DNA (10 mg/mL)

0.20 mL 50% Dextran Sulfate

0.02 mL 50x Denhardt’s

  - Alternative prehybridization buffer (4xSSC ; 50% deionized formamide) ;

Example. we need 10 ml prehyb.buffer

deionized formamide 		= 5 ml

20xSSC(4xSSC)		            = 2 ml

Add DEPC H2O			= 3 ml

2.  Add 0.5 mL to each slide and incubate w/o coverslips 1-1.30 hr at 370 C Or 2 hr at

RT in humid chamber.(Alternative incubate at least 10 min)

Hybridization ~ 30 min.

1.  Discard prehybridization buffer and rinse slides in 2x SSC and briefly dry prior to

hybridization.

2.  Add probe to prehyb. buffer at a concentration of 1:373

3.  Add probe to tissue sections. [300 ul each]

*4.  Denature tissue sections with probe for 10 min at 100 oC [*with a slide griddle                attached to a PTC-100 thermal controller]. Use hot plate for our lab.(Alternative)

5.  Cover tissue sections with Dnase-free coverslips.

6.  Incubate slides at 53 oC O/N in a humid chamber.

Stringency washes Day 2: Carefully remove cover slips from sections by immersing slides for 5-10 min in 2xSSC at RT

1.  Wash slides in 2x SSC twice for 15 min each at 40 oC.

2.  Wash in 1x SSC three times for 15 min at 40 oC.

3.  Wash in 0.5 x SSC for 15 min at 40 oC

4.  Equilibrate tissues in Buffer 1 (100 mM tris-HCl, 10 mM NaCl, pH 7.5) for 10 min

5.  Block tissues with Blocking Buffer (Buffer 1 + 2% sheep serum + 0.3% Triton X-

100)  for 1 hr at RT. {500ul each}

6. Never let the sections dry out.

Detection

1.  Prepare Alkaline phosphatase (AP)-labeled sheep anti-DIG antibody conjugate. Dilute

antibody 1:1000 in Buffer 1 + 1% Sheep serum + 0.3% Triton X-100.

2.  Add solution (0.5 -1 mL) to dry section and incubate without coverslip at RT

For 2 hours in a humid chamber.

3.  Rinse slides with Buffer 1 for 2 x 10 min.

4.  Rinse slides in Buffer 2 (100 mM tris-HCl, 100 mM NaCl, 50 mM MgCl2, pH 9.5 –

make sure!!) for 10 min.

5.  Prepare substrate by adding 45 ul NBT (nitroblue tetrazolium) + 35 ul BCIP to 10 mL of

Buffer2

6.  Incubate tissue sections with substrate solution 30 min -1 hour at RT in humid chamber.

7.  Rinse slides in 3x dH2O.

8.  Counter stain slides with 0.05% aqueous Bismarck Brown Y for 3 min.

9.  Rinse slides with dH2O, then 70% and 100% EtOH breifly.

10.Let slides air dry then permount.

Note:

To optimize the protocol the following variables can be modified:

1.  Concentration and length of solubilization with proteinase K.

2.  Dilution of probe and anti-DIG-AP antibody conjugate

3.  Temperature and length of incubation of anti-DIG antibody

4.  Temperature and stringency of washes

5.  Length of incubation of BCIP-NBT substrate. Signals should develop beginning at 30 min to 1 h. Do not incubate beyond 1 h.

Solutions

Tris Buffer

0.2 M Tris-HCl

2 mM CaCl

pH 7..2

PBS pH 7.4

1x SSC, pH 7.0		2x SSC, pH 7.2		20x SSC-store at 20C

0.15 M NaCl			0.3 M NaCl			3M NaCl

0.015 M sodium citrate	0.03 M sodium citrate		0.3M Na citrate

pH 7.0				pH 7.2				DI H2O

pH 7.2

Prehybridization Buffer

0.51 mL deionized formamide

0.2 mL 20x SSC

0.05 mL heat-denatured sperm DNA (at 10 mg/mL)

0.2 mL 50% dextran sulfate

0.02 mL 50x Denhardt’s solution

Add Poly A at 0.1 mg/mL

Buffer 1     :

100 mM Tris-HCl ;  Molar = g/MW; g = M x MW = 0.1M x 157.6 = 15.76g in 1 L H2O

10 mM NaCl         ; 0.01 x 58.44 = 0.5844 g in 1 L H2O

pH 7.5

Blocking Buffer

2% Sheep serum

0.3% Triton X-100 in Buffer 1

Antibody Solution

Alkaline Phosphate (AP)-labeled sheep anti-DIG antibody conjugate diluted 1:1000 in

Buffer 1 with 1% sheep serum + 0.3% Triton X-100

Buffer 2

100 mM tris-HCl

100 mM NaCl

50 mM MgCl2

pH 9.5

Day I

I. Abalone Rickettsia Okigonucleotide Primers: ** Optimum oligonucleotide conc. = 200 pm/ul

Primer                        Conc (pmole/ul)

RA 5-1			234

Have 234 pmole/ul (X) = want 200 pmole/ul x 200 ul?

X = 170.94 ul

RA 3-6			183	= 218.57  ul

RA 3-8			151	= 264.90  ul

RA 5-6			238	= 168.067 ul

Total					= 822.477 ul

Then:

RA 5-1: Have 822.477ul (X) = want 0.992 ul x 170.94ul

X  = 0.206 ul

RA 3-6:		          = 0.264 ul

RA 3-8                         = 0.319 ul

RA 5-6                         = 0.203 ul

Total                      = 0.992ul

II. Control group: = 5 ul  viral # 6 instead of the primers

4 ul Dnase free-water

Then add reagents (11 ul).

So the total of the Labeling procedure is 20 ul.

III. Permeabilization of tissues

- Wash tissues with 50 ug/ml of Proteinase K in Tris-buffer for 45 min at 37 C.

Working 50 ug/ml of Proteinase K in Tris-buffer.

Proteinase K: stock 20 mg/ml = 20,000 ug/ml

Example: Have 20 mg/ml (20,000 ug/ml) (X) want 50 ug/ml x 2,000 ul

X = 50 ug/ml x 2,000 ul/20,000 ug/ml

= 0.005 ml

Take Protein K	                = 5 ul

Tris buffer	 	   = 2,000 – 5 = 1,995 ul

** We Use about 15 ml/4 slides

** Then we want 15 ml of 50 ug/ml Proteinase- K in Tris-buffer

Have 20,000 ug/ml     (X)       want 50 ug/ml 15 ml

X			                 = 50 ug/ml x 15,000ul /20,000 ug/ml

Take Protein K	from stock             = 37.5 ul

Tris-buffer		                 =15,000 ul -37.5 ul = 14.97 ml

IV. Prehybridization buffer (Total volume = 1 ml/RXN),but we have to dilute the antibody in the prehybridization for the next step : so I would prepare for those.

Volume (ul)1 RXN	reagents				3.5 RXN (7 samples)

510 ul			deionized formamide			1,785 ul

200 ul			20X SSC				 700  ul

50  ul			heat-denatured sperm DNA (10 mg/mL)   175  ul

200 ul			50% Dextran Sulfate			  700  ul

20  ul			50X Denhardt’s				     70 ul

*** For 50% Dextran Sulfate : powder (on the shelf**)

Weigh 50 g in NanoH2O  100 ml

5 g in:::::::::::::::::::::10 ml

0.5 g in NanoH2O     1 ml

*** I want to stock 10 ml of 50% Dextran Sulfate

So I weigh 5 g in NanoH2O        10 ml

Then stir for 3 min.

V. Hybridization

Probe : Prehybridization buffer = 1:373 (Dolly recommend: 13:4850 )

Day 2

VI.Stringency washes

Blocking buffer  (5 ml/ 10 RXN) – I think , blocking buffer need per slide = 500 ul.

100 ul	2% sheep serum

15 ul	0.3% TritonX-100

4,885ul	Buffer 1

Total	   5 ml

VII. Detection

Antibody Buffer (7 RXN) =   7 ml		Buffer 1

=  21 ul		0.3% TritonX-100

=  70 ul		1% sheep serum

Dilute antibody 1: 1000 in Antibody buffer

Ex.     7 ul : 6,993 ul

VIII. Prepare substrate (ex.)

4 RXN			8 RXN

5 ml    Buffer 2		10 ml

22.5 ul  NBT   		45 ul

17.5 ul  BCIP 		35 ul

IX. Counter stain : Stock : Bismarck Brown Y 25 g as powder

I want 0.05% Bismarck Brown Y

0.05 g in Nano H2O  100 ml

Weigh 0.05 g of Bismarck Brown Y.

Dolly : Suggested to dilute Bismarck Brown Y

0.05% to 1:10 in PBS (Neutral pH)

God Bless ya :D